EMU41 Update 2006

Again Jean-Francois Garnier helpfully adds new features to his nice EMU41 emulator software : Up to now, for advanced applications like handling rom-images, creating own user code modules or machine code development, EMU41 emulated a 8KByte MLDL RAM page and the famous W&W RAM-Box. This virtual RAM-Box consists of two software selected 32 KByte banks = 64KByte RAM.

## ERAMCO 128KByte RSU

Now Jean-Francois also emulates the Eramco RSU RAM-Storage-Unit which supports four software selected 32 KByte banks = 128KByte RAM. The operating system for the W&W RAM-Box is stored in a 4KByte block. Similarly the Eramco operation system is stored in a 8KByte block and includes additionally commands for data storing applications and its own text editor. In my experience the 128KByte RSU from Eramco is the greatest "classic" RAM-Box solution, designed for the HP41 system. I have also heard rumours about an extended 256 KByte RSU solution (8 software selected banks) but have never seen this. To activate the Eramco 128Kbyte RSU to EMU41, edit the EMU41.INI file and add the following extensions :

8  96  es_rsu.dat    ; the 96k RAM = 4 banks of 24k in pages 8 to 13

14  8    rsu_os.rom  ; the RSU OS (supporting the multiple banks)

In this case the 8 KByte RSU operating system is addressed to port 14 and port 15. If so, as Jean-Francois explained, it would be possible to override RSU pages by plug in modules. In the following example we place the CCD-Module to port 8 and port 9

8  96  es_rsu.dat

14  8    rsu_os.rom

8  8    ccd1b.bin        ; override pages 8 & 9

When operating the RSU, remember that Eramco numbers the plug in port pages in an own way : W&W counts from page 8 up to page 15, identical to the Zenrom page numbering. In contrast to this, Eramco start with +1 for page 8, -1 for page 9 and ends with +4 for page 14 and -4 for page 15.

## RS232 support

Beside this great implementation of the Eramco 128KByte RSU (which works with its own bank switching mechanism) Jean-Francois also adds a virtual 
HP-IL/RS232 interface device to EMU41, supporting the serial COM port of the PC. Now EMU41 includes the following list of virtual HP-IL devices (taken from the EMU41.INI file) :

[DEVICES]

;

DISPLAY

LPRTER1

Now two serial devices are available for EMU41

HDRIVE1

;HDRIVE2

SERIAL1 1,E3

;SERIAL2 2,E3

;FDRIVE1       ;  insert a LIF DD floppy for correct function. Not available for modern PCs.

DOSLINK

XIL 300           ;   HP-IL/PC Interface Card - 700 default address / 300 address for WIN2000/NT/XP

The new SERIAL1 device acts like a virtual HP82164A HP-IL/RS232 interface for EMU41 and works as a gateway to you real HP41 handheld. Use this to transfer the X-Register contents, programs, data register blocks, data-files and matrix-files (CCD-Module) between EMU41 and your HP41 handheld computer. For your real HP41 you need the IL-Module, EXT-I/O and CCD-Module plus the HP-IL/RS232 interface HP82164A or CMT-RAM-DISC, wired to the serial COM port of the PC :

This new RS232 gateway solution between HP41 and EMU41 is handy for PC configurations that can not use the HP-IL/PC Interface Card : For example notebook- and laptop-PC´s and PC´s with motherboards without  an ISA-slot. As Jean-Francois explains, it would be also possible to add a RS232-to-USB Bus interface : With this expanded configuration you get a gateway solution for PC hardware that only supports the USB-Bus.

In contrast to the "classic" HP-IL/PC Interface Card gateway solution, the new RS232 application is limited to bidirectional program- and data transfer. Using the new RS232 gateway you are not able to control real HP-IL devices by EMU41. Controlling real HP-IL devices like the IL-Digitalvoltmeter or the IL-Plotter by EMU41 is only possible by using the HP-IL/PC Interface Card.

## HP41 software

In order to use the new RS232 gateway for interfacing EMU41 to your real HP41
I created a software pack plus associated user documentation. You can find this on the HPCC website. Because HP41 and EMU41 are running at different speeds (depending on the PC), and no COM data buffer exist on PC side for holding the transferred data for EMU41, we have a time critical application. Therefore the correct system configuration and time alignment is needed on the PC side, as described below..

For data transfer we execute the EXT-I/O Module commands OUTAN and INAN because only from this results a correct and save data transfer for this RS232 application. This two commands transfer a dummy-byte D followed by 7 data-bytes from the Alpha-Register of the sending device to the Alpha-Register of the receiving device. To exchange the 7 data-bytes between Alpha-Register and X-Register we use the (synthetic) commands STO M and RCL M. With CCD-Module you enter this commands as STO ∙ M and RCL ∙ M. For placing the leading dummy-byte D to the Alpha-Register we use the string "D6543210".

For program download from EMU41 to HP41 we use the standard commands OUTP and INP. Because of the critical timing needed for program upload from HP41 to EMU41 we can not execute the commands OUTP and INP. Instead we use the advanced commands PEEKB and POKEB from CCD-Module for the transfer of program bytes to the X-Register. Next STO M and RCL M transfer the program bytes to the Alpha-Register, and the commands OUTAN and INAN execute the RS232 transfer. Generally start the program upload from HP41 to EMU41 with GTO ∙ ∙ on the sending and receiving side. The EMU41 software routine RECVP for receiving program code acts as a self programming program. After the program code transfer is complete, RECVP writes the new .END. position to status register c. When RECVP is finished – again you need a manual PACK = GTO ∙ ∙. Beside the data or program values in the X-Register, the programs below needs some control numbers for controlling program loops or for calculating absolute addresses. All this numbers are managed completely by the stack lift – therefore no data register are needed.

Cable connection between serial COM 1 port and HP82164A

from J-F.Garnier

Shield			    	     Shield

1					1

2					2	transmitted data

3					3	received data

4					4	request to send

5					5	clear to send

6					6	data set ready

7					7	signal ground

8					8	carrier detect

9				         20	data terminal ready

9 pin				         25 pin

female				        female

connector				      connector

PC				     HP82164A

Cable connection between serial COM 1 port and CMT-RAM-DISC

Shield			    	     Shield

1					1

2					2	received data

3					3	transmitted data

4					4	data terminal ready

5					5	signal ground

6					6	data set ready

7					7	request to send

8					8	clear to send

9				          9

9 pin				         9 pin

female				        female

connector				      connector

PC				CMT-RAM-DISC

PC COM Port configuration

On PC side open the device manager for changing the settings needed for the serial COM 1 port :

9600 Byte per second

8 data bytes

no parity

1 stop bit

protocol type = None or XON/XOFF     depends on PC

extended settings :

using FIFO buffer                        depends on PC

buffer size = low … high             depends on PC

The first four settings you select with the device manager are fixed.. The needed protocol type and the definition of possible extended settings depending from your PC system. To reach the correct transfer of data please execute the short program tests in the next chapters :

EMU41 configuration

Open the EMU41.INI file and add the following extensions for activating the SERIAL1 or SERIAL2 feature. Furthermore you have to implement the IL-Module, the EXT-I/O and CCD-Module :

[MODULES]

;

; HP-41CX Rom-Modules :

0 12 rom41cx.bin

3  4 cxfcn2d.bin

5  8 cxtime2c.bin

;

; HP-IL Module : hard addressed module to port 6 + 7

6  4 ilprin2e.bin    ;   Printer              system rom XROM 29

7  4 ilmass1h.bin  ;   Mass Storage   system rom XROM 28

;

; HP-IL Accessory Modules :

10 4 extio1a.bin   ;      EXT-I/O        XROM 23

;

; Advanced-Modules :

8 8 ccd1b.bin      ;        CCD             XROM 09 + XROM 11

;

[DEVICES]

;

LPRTER1

1, = COM port 1

E3 = default communication parameters : 9600 baud, no parity, 1 stop bit, 8 data bits

HDRIVE1

HDRIVE2

SERIAL1 1,E3

;SERIAL2 2,E3

;FDRIVE1          ;   insert a LIF DD floppy for correct function. Not available for modern PCs.

DOSLINK

XIL 300           ;   HP-IL/PC Interface Card - 700 default address,

;                                                                       300 address for WIN2000/NT/XP

;

Software Commands for program- and data-transfer

The following software commands are available for program and data transfer between HP41 and EMU41 using the RS232 gateway :

SENDX	Send X-Register

SENDP	Send user code program

SENDR	Send main memory register block

SENDF	Send X-Memory data-file

SENDM	Send CCD-Module matrix-file

RECVX	Receive X-Register

RECVP	Receive user code program

RECVR	Receive main memory register block

RECVF	Receive X-Memory data-file

RECVM	Receive CCD-Module matrix-file

Starting with a first transfer test

For starting a first transfer test enter the program codes for SENDX and RECVX including the belonging subroutines LBL 20 (initialisation) and LBL 30 (waiting routine) and LBL 41 (end routine) to HP41 and to EMU41. Note that the program codes for SENDP and for RECVP, for LBL 20 and LBL 30 are different for HP41 and for EMU41.

Now start a X-Register upload from HP41 to EMU41 : Execute RECVX on EMU41. Next enter a number to the HP41 X-Register and execute SENDX on the HP41. In the best case now you find this number also in the EMU41 screen. If not change some settings on PC side : May be LBL 30 runs in an endless loop, than your PC is not able to use this subroutine – in this case remove it. Next change the settings needed for the COM port in the device manager as described above. Last you are able to speed up EMU41 by pressing the tab-key. This would be helpfully for older PC systems. If X-Register data upload from HP41 to EMU41 works correctly go on with a X-Register download from EMU41 to HP41.

Register-Block transfer

In the next step you execute a register-block upload from HP41 main memory to EMU41 main memory. Enter a control number bbb.eee on both sides, bbb is the first register and eee is the last register you want to transfer. Begin with an upload of only 10 data-registers from HP41 to EMU41 : First execute RECVR on EMU41 than SENDR on the HP41. Generally the data register upload from HP41 to EMU41 needs no time alignment because the PC waits for data coming from HP82164A.

If the results on EMU41 side are correct, execute a register-block download from EMU41 to HP41. Depending on your PC system you have to modify LBL 50 which works as a software delay routine : When your PC runs too fast, you have to insert a larger number of PSE commands. This slows down the sending routines of EMU41. You reach the correct time alignment when both routines SENDR and RECVR on both sides are finished at nearly the same time. Check this time alignment also by downloading a greater number of data registers from EMU41 to HP41. The following routines are working with the identical time alignment.

Data-File transfer

For data-file transfer create the needed X-Memory data-file or CCD-Module matrix-file with correct size on the receiving HP41 or EMU41. Next enter the file name to Alpha-Register on both HP41 and EMU41 Now start the receive routine RECVF or RECVM on the receiving device. Last start the send routine SENDF or SENDM on the sending device.

Program transfer

Program download form EMU41 to HP41 is easy, first start RECVP on the HP41 than execute SENDP on EMU41 and enter the program name at the Alpha prompt followed by R/S.

If you want to upload a program from HP41 to EMU41, first check if enough storage register exist to accommodate the transferred program on the receiving EMU41 side by entering the PRGM mode,. If not enough register exist change SIZE or delete an other program from main memory. When you make not enough memory space available, than EMU41 runs to a MEMORY LOST. Next execute GTO ∙ ∙ on both sides. Than start RECVP on EMU41. Now start SENDP on HP41, enter the program name and press R/S. When the transfer is finished you have to execute a manual PACK = GTO ∙ ∙ on EMU41 !

Software for HP41 and HP82164A

01_LBL "RS232"

02_LBL "SENDX			send X-Register

03 XEQ 20				initialisation of the HP82164A

04 "D6543210"			loaf dummy Bytes to Alpha

05 STO M				STO ∙ M replaces dummy Bytes with

06 7					X-Register contents

07 OUTAN				send Alpha-Register

08 RDN

09 GTO 41				end routine

10 RTN

11_LBL "SENDP"			first execute GTO ∙ ∙ than send program

12 XEQ 20				initialisation of the HP82164A

13 "PROG NAME ?"		asks the program name

14 AON

15 PROMPT

16 AOFF

17 PHD				absolute address of the first program Byte

18 ENTER^

19 ENTER^

20 PPLNG

21 2

22 -

23 "D6543210"

24 STO M

25 7

26 OUTAN				send program length in Bytes

27 RDN

28 CHS

29 A+B

30 X<>Y

31_LBL 03

32 PEEKB

33 STO M				STO ∙ M

34 7

35 OUTAN				send program Byte

36 RDN

37 RDN

38 A-

39 X≠Y?

40 GTO 03

41 GTO 40

42 RTN

43_LBL "SENDR"			send data register block

44 XEQ 20

45 "D6543210"

46_LBL 00

47 RCL IND X

48 STO M				STO ∙ M

49 7

50 OUTAN

51 RDN

52 RDN

53 ISG X

54 GTO 00

55 GTO 40

56 RTN

57_LBL "SENDF"			send X-Memory data-file

58 CLX

59 SEEKPTA

60 XEQ 20

61 "D6543210"

62_LBL 01

63 SF 25

64 GETX

65 FC? 25

66 GTO 39

67 PSE

68 STO M				STO ∙ M

69 7

70 OUTAN

71 GTO 01

72 RTN

73_LBL "SENDM"			send matrix-file

74 1.001

75 IJ=A

76 XEQ 20

77 "D6543210"

78_LBL 02

79 SF 25

80 C>+

81 FC? 25

82 GTO 39

83 PSE

84 STO M				STO ∙ M

85 7

86 OUTAN

87 GTO 02

88 RTN

89_LBL "RECVX"			receive X-Register

90 XEQ 20

91 XEQ 30

92 CLA

93 7

94 INAN				get data from HP84164A

95 RCL M				RCL ∙ M

96 GTO 41

97 RTN

98_LBL "RECVP"			receive program from EMU41

99 XEQ 20

100 XEQ 30

101 INP

102 GTO 40

103 RTN

104_LBL "RECVR"			receive data register block

105 XEQ 20

106 XEQ 30

107_LBL 10

108 CLA

109 7

110 INAN

111 RDN

112 RCL M				RCL ∙ M

113 STO IND Y

114 RDN

115 ISG X

116 GTO 10

117 GTO 40

118 RTN

119_LBL "RECVF"			receive X-Memory data-file

120 CLX

121 SEEKPTA

122 XEQ 20

123 XEQ 30

124_LBL 11

125 CLA

126 7

127 INAN

128 RCL M				RCL ∙ M

129 SF 25

130 SAVEX

131 FS? 25

132 GTO 11

133 GTO 40

134 RTN

135_LBL "RECVM"			receive matrix-file

136 1.001

137 IJ=A

138 XEQ 20

139 XEQ 30

140_LBL 12

141 CLA

142 7

143 INAN

144 RCL M				RCL ∙ M

145 SF 25

146 >C+

147 FS? 25

148 GTO 12

149 GTO 40

150 RTN

151_LBL 20				initialisation routine for the HP82164A     )**

152 "HP82164A"

153 FINDID

154 SELECT

155 MANIO

156 CF 17

157 REMOTE

158 "LI5"				disable data terminal ready control signal line

159 OUTA				(R04-3)

160 "C0"				no software protocol (R11-3,2,1,0)

161 OUTA

162 NOTREM

163 RDN

164 RTN

165_LBL 30				waiting for data from HP82164A

166 ENTER^

167 CLX

168 X<>FIO

169_LBL 31

170 STAT

171 2

172 ATOXX

173 X<>FIO

174 RDN

175 FC? 02

176 GTO 31

177 X<>FIO

178 RDN

179 RTN

180_LBL 39				transfers dummy data

181 CLX

182 STO M

183 7

184 OUTAN

185_LBL 40				end routine clears Alpha and X-Register

186 CLX

187_LBL 41

188 AUTOIO

189 CLA

190 END

Software for EMU41

01_LBL "COM"

02_LBL "SENDX			send X-Register

03 XEQ 20				initialisation SERIAL1

04 "D6543210"			loaf dummy Bytes to Alpha

05 STO M				STO ∙ M replaces dummy Bytes with

06 7					X-Register contents

07 OUTAN				send Alpha-Register

08 RDN

09 GTO 41				end routine

10 RTN

11_LBL "SENDP"			send program

12 XEQ 20				initialisation SERIAL1

13 "PROG NAME ?"		asks for the program name

14 AON

15 PROMPT

16 AOFF

17 OUTP

18 GTO 40

19 RTN

20_LBL "SENDR"			send data register block

21 XEQ 20

22 "D6543210"

23_LBL 00

24 RCL IND X

25 XEQ 50

26 STO M				STO ∙ M

27 7

28 OUTAN

29 RDN

30 RDN

31 ISG X

32 GTO 00

33 GTO 40

34 RTN

35_LBL "SENDF"			send X-Memory data-file

36 CLX

37 SEEKPTA

38 XEQ 20

39 "D6543210"

40_LBL 01

41 SF 25

42 GETX

43 XEQ 50

44 FC? 25

45 GTO 39

46 STO M				STO ∙ M

47 7

48 OUTAN

49 GTO 01

50 RTN

51_LBL "SENDM"			send matrix-file

52 1.001

53 IJ=A

54 XEQ 20

55 "D6543210"

56_LBL 02

57 SF 25

58 C>+

59 FC? 25

60 GTO 39

61 XEQ 50

62 STO M				STO ∙ M

63 7

64 OUTAN

65 GTO 02

66 RTN

67_LBL "RECVX"			receive X-Register

68 XEQ 20

69 XEQ 30

70 CLA

71 7

72.INAN				get data from COM

73 RCL M				RCL ∙ M

74 GTO 41

75 RTN

76_LBL "RECVP"			receive program

78 16

79 WSIZE

80 13.1				calculates the absolute address of the

81 PEEKB				first free Byte behind the last program

82 15					in main memory

83 AND

84 256

85 *

86 13.0

87 PEEKB

88 X<>Y

89 RDN

90 +					absolute address

91 ENTER^

92 ENTER^

93 0.2

94 +

95 PEEKB

96 S>

97 7

98 AND				number of Bytes

99 RCL Z

100 ENTER^

101 0.1

102 +

103 PEEKB				number of Register

104 7

105 *

106 RCL Z

107 +

108 X<>Y

109 INT

110 X<>Y

111 A+B

112 A-					absolute address of the first free Byte

113 XEQ 20

114 XEQ 30

115 ENTER^

116 CLA

117 7

118 INAN				read length of the program in Bytes

119 RDN

120 ENTER^

121 RCL M				RCL ∙ M

122 CHS

123 A+B

124 X<>Y

125_LBL 04

126 CLA

127 7

128 INAN				read program Byte from COM

129 RCL M				RCL ∙ M

130 X<>Y

131 RDN

132 POKEB				write program Byte to main memory

133 RDN

134 A-

135 X≠Y?

136 GTO 04

137 INT				add final END

138 1

139 -

140 "D____ _-"			"D 000 000 000 000 192 000 045"     )*

141 RCL M				RCL ∙ M

142 POKER

143 RDN				place new .END. position

144 13.1				to status register c

145 PEEKB

146 240

147 AND

148 256

149 *

150 RCL Z

151 +

152 "D"

153 XTOAH

154 13.0

155 ATOXR

156 POKEB

157 13.1

158 ATOXR

159 POKEB

160 GTO 40

161 RTN				after RECVP execute manual PACK = GTO ∙ ∙

162_LBL "RECVR"			receive data register block

163 XEQ 20

164 XEQ 30

165_LBL 10

166 CLA

167 7

168 INAN

169 RDN

170 RCL M				RCL ∙ M

171 STO IND Y

172 RDN

173 ISG X

174 GTO 10

175 GTO 40

176 RTN

177_LBL "RECVF"			receive X-Memory data-file

178 CLX

179 SEEKPTA

180 XEQ 20

181 XEQ 30

182_LBL 11

183 CLA

184 7

185 INAN

186 RCL M				RCL ∙ M

187 SF 25

188 SAVEX

189 FS? 25

190 GTO 11

191 GTO 40

192 RTN

193_LBL "RECVM"			receive matrix-file

194 1.001

195 IJ=A

196 XEQ 20

197 XEQ 30

198_LBL 12

199 CLA

200 7

201 INAN

202 RCL M				RCL ∙ M

203 SF 25

204 >C+

205 FS? 25

206 GTO 12

207 GTO 40

208 RTN

209_LBL 20				initialisation SERIAL1

210 "SERIAL1"

211 FINDID

212 SELECT

213 MANIO

214 CF 17

215 RDN

216 RTN

217_LBL 30				waiting for data from COM port

218 INSTAT				not any PC support this subroutine

219 RDN				than delete program lines 218 to 222

220 FC? 00

221 GTO 30

222 CF 00

223 RTN

224_LBL 39				transfer dummy data

225 CLX

226 STO M

227 7

228 OUTAN

229_LBL 40				end routine clears Alpha and X-Register

230 CLX

231_LBL 41

232 AUTOIO

233 CLA

234 RTN

235_LBL 50				time delay routine for slow speed PC

236 PSE				number of needed PSE commands depends on PC

237 END				and EMU41 speed mode

)* For   entering   the   synthetic   text   line   "D  000  000  000  000  192  000  045"  use    the

CCD-Module :  Go to the  ALPHA  mode,  deactivate  USER , press  SHIFT  ENTER  and

you  get  the  prompt  : _ _ _   for  the  input  of  the  decimal  values.

)** Set HP-IL/RS232 interface HP82164A to DTE mode (check the internal jumper position).

## New EMU41 applications

Great thanks to Jean-Francois for expanding EMU41 with advanced RSU storage and RS232 gateway features. Jean-Francois has also added this new RS232 extension to EMU71. This is an encouraging result from a discussion during the last Allschwil meeting (HP-Collection) in Switzerland. In my own opinion EMU41 is the world´s leading HP41 emulator software because :

EMU41 runs on different HP handheldcomputer- and PC platforms

EMU41 allows the rom-image formats .bin and .rom

EMU41 includes plug in modules and bank switched modules

EMU41 includes HEPAX, MBK-Profiset, RAM-BOX and RSU support

EMU41 includes a gateway for data exchange to other PC applications

EMU41 includes different gateway solutions for interfacing your real HP41

EMU41 includes some virtual IL devices like printer and mass storage

EMU41 includes virtual tapes for email exchange of data, programs, rom-images

EMU41 is able to control real HP-IL devices

May this development will result some more HP41 applications : For example an umbilical cord from EMU41, using RS232 + USB, to the MLDL2000 unit from Meindert Kuipers ?