HP-41 // RS232 --------- RS232 // HP-41

Transfer of programs, data register blocks, data-files, matrix-files, ASCII-files and NNN-blocks (Non Normalised Number) between two HP-41 handheld computers by using two HP-IL/RS232 interfaces HP82164A.

Program needs IL-Module, EXT-I/O and CCD-Module (and optional the ZENROM).

Create the needed data-, matrix-, ASCII- file with correct size on the receiving HP41. Now enter the block number to X-Register or the file name to Alpha-Register on both HP41. First start the receive routine on the receiving HP41. Last start the send routine on the sending HP41.

Both HP-IL/RS232 interfaces HP82164A set to DTE mode (check the internal jumper position). Below you find the needed wiring between the two HP82164A interface devices :

Shield			    	     Shield

1					1	???

2					2	transmitted data

3					3	received data

4					4	request to send

5					5	clear to send

6					6	data set ready

7					7	signal ground

8					8	carrier detect

20				         20	data terminal ready

25 pin				      25 pin

female				      female

connector				    connector

The actual HP41 software below includes the following limits :  Data register blocks and data-files containing numbers (like -12,34E-56) are transferred correctly. Pure alpha text (like abcXYZ) are also transferred correctly. But alpha text mixed with numbers (like ABC123) would be reduced to the number on the receiving side. For transferring this correctly use the NNN transfer routines. Furthermore the NNN transfer routines are able to transfer any possible byte value from 000 up to 255 correctly. By using the ZENROM on the receiving side you are able to speed up the transfer time for NNN´s. For ASCII file transfer the record length is limited to 24 signs/characters, the maximally length of the Alpha-Register.

10:22  18.01.2006

01_LBL "SENDP"		Send USER code program

02 XEQ 20			Asks for the program name

03 "PROG NAME ?"

04 AON

05 PROMPT

06 AOFF

07 OUTP

08 GTO 40

09 RTN

10_LBL "SENDR"		Send Main Memory register block

11 XEQ 20			Start with

12_LBL 00			control number bbb.eee in X-Register :

13 CLA				bbb = first register

14 ARCL IND X			eee = last register

15 OUTA

16 ISG X

17 GTO 00

18 GTO 40

19 RTN

20_LBL "SENDF"		Send X-Memory Datafile.

21 CLX				Start with filename in Alpha-Register

22 SEEKPTA

23 XEQ 20

24_LBL 01

25 CLA

26 SF 25

27 GETX

28 FC? 25

29 GTO 40

30 ARCL X

31 OUTA

32 GTO 01

33 RTN

34_LBL "SENDM"		Send CCD-Module Matrix file

35 1,001				Start with filename in Alpha-Register

36 IJ=A

37 XEQ 20

38_LBL 02

39 CLA

40 SF 25

41 C>+

42 FC? 25

43 GTO 40

44 ARCL X

45 OUTA

46 GTO 02

47 RTN

48_LBL "SENDA"		Send X-Memory ASCII-File

49 CLX				Start with filename in Alpha-Register

50 SEEKPTA

51 XEQ 20

52_LBL 03

53 CLA

54 SF 25

55 GETREC

56 FC? 25

57 GTO 40

58 OUTA

59 GTO 03

60 RTN

61_LBL "SENDNNN"		Send Main Memory NNN-Block

62 XEQ 20			Start with

63 ENTER^			control number bbb.eee in X-Register :

64 ENTER^			bbb = first register

65 512				eee = last register

66 SIZE?

67 -

68 STO Y

69 1000

70 /

71 +

72 +

73_LBL 04

74 CLA

75 PEEKR

76 DCD				Decodes NNN to Alpha text stream

77 OUTA

78 PSE

79 PSE				Wait cycle nearly identical to

80 PSE				running time of CDE subroutine

81 PSE

82 PSE

83 PSE				Program lines 78 to 85 are not needed

84 PSE				when using the fast CODE command from

85 PSE				ZENROM

86 RDN

87 ISG X

88 GTO 04

89 GTO 40

90 RTN

91_LBL "RECVP"		Receive User code program

92 XEQ 30			No input needed

93 INP

94 GTO 40

95 RTN

96_LBL "RECVR"		Reveive Main Memory register block

97 XEQ 30			Start with

98_LBL 10			control number bbb.eee in X-Register :

99 CLA				bbb = first register

100 INA				eee = last register

101 ENTER^

102 CF 22

103 ANUM

104 FS? 22

105 STO IND Y

106 FC? 22

107 ASTO IND Y

108 RDN

109 ISG X

110 GTO 10

111 GTO 40

112 RTN

113_LBL "RECVF"		Receive X-Memory Datafile

114 CLX				Start with filename in Alpha Register

115 SEEKPTA

116 XEQ 30

117_LBL 11

118 CLA

119 INA

120 CF 22

121 ANUM

122 FS? 22

123 STO X

124 FC? 22

125 ASTO X

126 SF 25

127 SAVEX

128 FS? 25

129 GTO 11

130 GTO 40

131 RTN

132_LBL "RECVM"		Receive CCD-Module Matrix file

133 1,001				Start with filename in Alpha-Register

134 IJ=A

135 XEQ 30

136_LBL 12

137 CLA

138 INA

139 CF 22

140 ANUM

141 FS? 22

142 STO X

143 FC? 22

144 ASTO X

145 SF 25

146 >C+

147 FS? 25

148 GTO 12

149 GTO 40

150 RTN

151_LBL "RECVA"		Receive X-Memory ASCII-File

152 CLFL				Start with filename in Alpha-Register

153 XEQ 30

154_LBL 13

155 CLA

156 INA

157 ALENG

158 X=0?				Test : Alpha-Register empty

159 GTO 40

160 SF 25

161 APPREC

162 FS? 25			Test : End Of File

163 GTO 13

164 GTO 40

165 RTN

166_LBL "RECVNNN"		Receive Main Memory NNN-Block

167 XEQ 30			Start with

168 ENTER^			control number bbb.eee in X-Register :

169 ENTER^			bbb = first register

170 512				eee = last register

171 SIZE?

172 -				Program version with CDE subroutine

173 STO Y				from CCD-module manual

174 1000				needs data register 00

175 /

176 +				Fast program version with

177 +				CODE command from ZENROM :

178 STO 00			178_LBL 14

179_LBL 14			179 CLA

180 CLA				180 INA

181 INA				181 CODE

182 XEQ "CDE"			182 POKER

183 RCL 00			183 RDN

184 RCL M				184 ISG X

185 POKER				185 GTO 14

186 ISG 00			186 GTO 40

187 GTO 14			187 RTN

188 GTO 40			About incompatibility of CCD-Module

189 RTN				and ZENROM refer to CCD-Module manual

190_LBL 20			Initialisation of the

191 AUTOIO			sending HP-IL/RS232 Interface

192 "HP82164A"

193 FINDID

194 SELECT

195 MANIO

196 CF 17

197 CLDEV

198 RDN

199 RTN

200_LBL 30			Initialisation of the

201 AUTOIO			receiving HP-IL/RS232 Interface

202 "HP82164A"

203 FINDID

204 SELECT

205 MANIO

206 CF 17

207 CLRDEV

208 RDN

209 ENTER^

210 CLX

211 X<>FIO

212_LBL 31

213 STAT				receive buffer contains data

214 2

215 ATOXX

216 X<>FIO

217 RDN

218 FC? 02

219 GTO 31

220 X<>FIO

221 RDN

222 RTN

223_LBL 40			End routine

224 AUTOIO

225 CLX

226 CLA

227 RTN

228_LBL "CDE"			Subroutine from CCD-Module Manual :

229 CLX				Codes Alpha text stream to NNN

230 WSIZE

231 6,5

232 SF 22				Instead CDE you can use the fast

233_LBL 50			CODE command from the ZENROM

234 5

235 PEEKB

236 ABSP

237 X<>Y

238 RDN

239 57

240 -

241 X>0?

242 2

243 X<=0?

244 9

245 +

246 X<0?

247 CLX

248 FS?C 22

249 GTO 50

250 16

251 *

252 +

253 POKEB

254 SF 22

255 CLX

256 5,5

257 X<>Y

258 A-

259 X≠Y?

260 GTO 50

261 RCL M				RCL ∙ M

262 END				Program length = 594 Bytes

01_LBL "IA"				Utility software routine for

02 CLFL					loading ASCII file with text

03_LBL 00				File name in Alpha-Register

04 CLA

05 "TEXT ?"

06 CF 23

07 AON

08 PROMPT

09 AOFF

10 FC? 23

11 RTN

12 SF 25

13 APPREC

14 FS? 25

15 GTO 00

16 RTN

17_LBL "OA"				Utility software routine for

18 CLX					displaying ASCII file contents

19 SEEKPTA				File name in Alpha-Register

20 CLA

21_LBL 01

22 GETREC

23 AVIEW

24 PSE

25 CLA

26 GTO 01

27 END					Program length = 55 Bytes

HP-41 // RS232 ---- RS232 // HP-41

software was programmed and tested by

Christoph Klug

Körnerstrasse 47B

31141 Hildesheim

Germany

© January 2006